--  TAPELIB --

BY  SAM J. SINGER

(714) 780-8853

I.  INTRODUCTION

TAPELIB IS A GENERAL CASSETTE TAPE LIBRARY MANAGER FOR

CP/M. IT INCORPORATES MOST OF THE FEATURES OF THE CP/M PIP UTILITY.

ANY CP/M FILE OR GROUP OF FILES MAY BE COPIED FROM TAPE TO DISK

OR FROM DISK TO TAPE.

IN ADDITION TO THE USUAL PIP COPY OPERATIONS, TAPELIB

PERMITS VERIFICATION OF TAPE FILES AGAINST DISK FILES AND DIRECT

EXECUTION OF BINARY PROGRAMS FROM TAPE.  TAPE FILES ARE NAMED AND

SPACE IS PROVIDED IN THE TAPE COPY FOR EXTENSIVE COMMENTS.

FILES MAY BE DISPLAYED ON THE CONSOLE DURING COPY OPERATIONS

FOR VISUAL VERIFICATION .  THE PROGRAM CONTAINS EXTENSIVE ERROR

CHECKS, AND CONSOLE MESSAGES ARE UNABBREVIATED AND QUITE SPECIFIC.

EVERY EFFORT HAS BEEN MADE TO MAKE THE PROGRAM GENERAL AND EASY TO

USE. THE PROGRAM HAS BEEN OPTIMIZED TO PROVIDE THE FASTEST POSSIBLE

DATA TRANSFER RATES YET STILL MAINTAIN GOOD RELIABILITY.

II.  HARDWARE REQUIREMENTS

TAPELIB REQUIRES AN 8080 OR Z-80 BASED COMPUTER SYSTEM

RUNNING THE CP/M OPERATING SYSTEM, AT LEAST ONE FLOPPY DISK DRIVE,

A CASSETTE RECORDER AND TARBELL CASSETTE INTERFACE AND A CONSOLE

DEVICE.  THE DISPLAY DURING COPY OPERATIONS REQUIRES THE TERMINAL

TO OPERATE AT A MINIMUM OF 4800 BAUD IF THIS FEATURE IS TO BE USED.

III.  PROGRAM OPERATION

TAPELIB OPERATES AS A TRANSIENT FILE UNDER CP/M, AND

REQUIRES SLIGHTLY MORE THAN 4K OF MEMORY FOR CODE. THE REMAINDER

OF THE AVAILABLE MEMORY WILL BE USED FOR FILE BUFFERS.

DURING DISK TO TAPE COPIES THE PROGRAM WILL FIRST ATTEMPT

TO READ THE ENTIRE FILE INTO MEMORY AND WRITE IT ON TAPE AS A

SINGLE RECORD. IF THERE IS INSUFFICIENT ROOM FOR THE ENTIRE FILE,

OR IF THE FILE CONTAINS MORE THAN ABOUT 14000 BYTES THE PROGRAM

WILL WRITE THE FILE ON TAPE IN 1K BLOCKS. A BLOCKED TAPE WILL ALSO

BE PRODUCED IF THE BLOCK OPTION IS USED. THIS OPTION IS USEFUL IF

TAPES ARE TO BE TRANSFERED FROM ONE SYSTEM TO ANOTHER OF A DIFFERENT

SIZE. A MESSAGE WILL BE DISPLAYED BEFORE A BLOCKED TAPE IS WRITTEN.

BLOCKED TAPES REQUIRE ABOUT 25 PER CENT MORE TIME TO WRITE.

EACH TAPE FILE IS HEADED BY A 128 BYTE RECORD CONTAINING

THE FILE NAME. MOST OF THE REMAINDER OF THE 128 BYTES IS AVAILABLE

FOR COMMENTS. THE COMMENTS ARE DELETED DURING TAPE TO DISK COPIES.

FILES ARE DISPLAYED ON THE TERMINAL DURING ALL COPY

OPERATIONS UNLESS THE NODISPLAY OPTION IS USED. CHARACTERS ARE

DISPLAYED IN ASCII IN 64 CHARACTER LINES WITH NONDISPLAYABLE

CHARACTERS CONVERTED TO PERIODS.

DURING TAPE TO DISK COPY OPERATIONS A SEARCH IS MADE FOR

THE FILE NAME OR GROUP OF NAMES AND ONLY THE REQUESTED FILES

ARE COPIED. HOWEVER, THE HEADER RECORD CONTAINING THE FILE NAME

WILL BE DISPLAYED FOR EACH FILE ON THE TAPE. THIS PERMITS SCANNING

THE TAPE TO DETERMINE ITS CONTENTS, SINCE TAPES HAVE NO DIRECTORIES.

WHEN REQUESTED FILES ARE FOUND, THE ENTIRE CONTENTS WILL BE DISPLAYED.

IV.  TAPELIB COMMANDS

TAPELIB MAY BE RUN BY ONE OF TWO METHODS. TYPE:

TAPELIB

TAPELIB <COMMAND STRING>

ALL COMMANDS ARE TERMINATED BY A CARRIAGE RETURN. IF THE FIRST METHOD IS

USED THE PROGRAM RESPONDS WITH A TITLE AND AN * PROMPT. THE COMMAND STRING

MAY THEN BE ENTERED. THE SECOND FORM EXECUTES THE COMMAND STRING

IMMEDIATELY.

IF THE FIRST FORM OF THE CALL IS USED THE PROGRAM WILL RETURN TO

THE INPUT SECTION AFTER COMPLETION OF THE COMMAND. THE PROMPT WILL BE

DISPLAYED AND ANOTHER COMMAND EXECUTED WITHOUT RELOADING THE PROGRAM.

THE OTHER FORM WILL RETURN TO CP/M AFTER COMPLETION OF THE COMMAND.

ANY COMMAND MAY BE TERMINATED BY TYPING CONTROL C. THE FIRST CONTROL C

RETURNS TO THE INPUT SECTION OF THE PROGRAM. A SECOND CONTROL C RETURNS

BACK TO CP/M.

THE COMMAND STRING HAS THE FOLLOWING FORM:

DESTINATION:=SOURCE  <OPTIONS> <COMMENT>

THE SOURCE AND DESTINATION MAY BE EITHER A FILE NAME AND TYPE, A

DEVICE NAME OR SPECIAL FUNCTION. DEVICE NAMES AND FUNCTIONS MUST BE FOLLOWED

BY A COLON. THE FOLLOWING TABLE GIVES ACCEPTABLE NAMES AND THEIR

INTERPRETATION.

DEVICE/FUNCTION TABLE

TAPE:		THE CASSETTE TAPE

DISK:		THE CURRENTLY LOGGED DISK

A:		DISK A

B:		DISK B

RUN:		LOAD TAPE TO MEMORY AND EXECUTE

SYNC:		WRITE SYNC STREAM ON TAPE

ANY CP/M LEGAL FILE NAME INCLUDING AMBIGUOUS FILE NAMES MAY BE

USED WITH TAPELIB. THE FOLLOWING ARE EXAMPLES OF ACCEPTABLE NAMES

XXXX.YYY	FILENAME.TYP

*.COM		TAPELIB.???

*.*		A:MYPROG.COM

B:NEWNAME.ASM	B:????????.ASM

NOTE THAT THE DISK NAME MAY BE SPECIFIED AS PART OF A FILE NAME.

THE OPTIONS AVAILABLE AND THEIR FUNCTIONS ARE LISTED IN THE

OPTION TABLE WHICH FOLLOWS.

OPTION TABLE

NODISPLAY		TURN OFF DISPLAY FEATURE (FOR SLOW TERMINALS)

BLOCK			BLOCK FILES ON TAPE IF FILE WILL NOT FIT

IN 16K CP/M SYSTEM

VERIFY			COMPARE TAPE FILE WITH DISK FILE OF SAME NAME

RENAME			RENAME FILE ON DISK AFTER COPY FROM TAPE

COMMAND STRINGS CAN BE CONSTRUCTED FROM VARIOUS COMBINATIONS OF

THE NAMES AND OPTIONS LISTED ABOVE. IN GENERAL MORE THAN ONE OPTION CAN

BE INCLUDED IN ANY COMMAND STRING. IF COMMENTS OR OPTIONS ARE INCLUDED

IN A CASE WHERE THEY ARE NOT MEANINGFUL, THEY WILL BE IGNORED.

V.  EXAMPLES OF COMMANDS

DISK TO TAPE

TAPE:=FILENAME.TYP	'OPTIONAL COMMENTS'

TAPE:=B:*.COM      RECORDED DEC 26 1977

TAPE:=A:MYPROG.ASM    NODISPLAY

TAPE TO DISK

DISK:=*.COM

A:=GAME.BAS   RENAME NEWGAME.BAS

B:=TESTPROG.ASM     VERIFY

OTHER COMMANDS

TAPE:=SYNC:		(WRITE 6 MIN OF SYNC CODE)

RUN:=ANYPROG.COM	(LOAD FROM TAPE AND EXECUTE)

THE FORMAT OF COMMAND STRINGS IS FREE EXCEPT FOR THE SOURCE

AND DESTINATION. THE EQUAL SIGN AND COLONS ARE REQUIRED AND NO SPACES

ARE PERMITTED IN THE SOURCE OR DESTINATION. THE OPTIONS AND COMMENTS

MAY BE TYPED ANYWHERE ON THE INPUT LINE. INTERVENING SPACES WILL BE

SKIPPED OVER.

TO RENAME A FILE DURING A TAPE TO DISK COPY ENTER THE WORD

RENAME FOLLOWED BY THE NEW FILE NAME AND TYPE ANYWHERE ON THE INPUT

LINE FOLLOWING THE SOURCE. THIS FEATURE IS USEFUL IF A FILE WITH THE

SAME NAME ALREADY EXISTS ON THE DISK.

TO VERIFY A FILE OR GROUP OF FILES EXECUTE A TAPE TO DISK COPY

OPERATION WITH THE WORD VERIFY ANYWHERE FOLLOWING THE SOURCE. THE COPY

OPERATION WILL NOT ACTUALLY TAKE PLACE. INSTEAD THE FILE WILL BE READ

FROM TAPE AND COMPARED BYTE BY BYTE WITH THE FILE OF THE SAME NAME ON

THE DISK SPECIFIED. IF THE FILES ARE NOT THE SAME THE PROGRAM DISPLAYS

VERIFY ERROR

IF THE FILES ARE THE SAME THE PROGRAM WILL DISPLAY:

VERIFIED SUCCESSFULLY

TO RUN A PROGRAM FROM TAPE TYPE RUN: AS THE DESTINATION. THE

PROGRAM CHECKS THE FILE TYPE BEFORE EXECUTION BEGINS AND WILL DISPLAY

AN ERROR MESSAGE IF THE FILE IS NOT TYPE 'COM'. COM FILES ARE MOVED TO

STARTING LOCATION 100H BEFORE BEING EXECUTED.

VII.  PROMPTS AND ERROR MESSAGES

TAPELIB ASSUMES THAT ONLY AN INEXPENSIVE NON CONTROLED CASSETTE

TAPE RECORDER IS BEING USED. THE PROGRAM HALTS AT THE PROPER TIMES AND

DISPLAYS INSTRUCTIONS FOR STARTING AND STOPPING THE TAPE. FOR EXAMPLE

TO COPY DISK TO TAPE THE PROGRAM WILL PAUSE AND DISPLAY:

START CASSETTE TAPE <<RECORD>> - TYPE CARRIAGE RETURN

FOR TAPE TO DISK OPREATIONS THE MESSAGE WILL BE:

START CASSETTE TAPE - TYPE CARRIAGE RETURN

THE TAPE SHOULD BE STARTED IN THE PROPER MODE BEFOR TYPING THE CARRIAGE

RETURN.

TAPELIB IS EXTENSIVELY TRAPPED FOR ERRORS. IF AN ERROR OCCURS

THE PROGRAM WILL DISPLAY AN ERROR MESSAGE AND PAUSE TO ALLOW THE ERROR

TO BE CORRECTED IF POSSIBLE. THE FOLLOWING TABLE CONTAINS ERROR MESSAGES,

THEIR PROBABLE CAUSE AND POSSIBLE CORRECTION.

ERROR TABLE

1.  ERROR IN FILE NAME		INCORRECT FILE NAME - REENTER NAME

2.  NO SUCH DESTINATION		INCORRECT DESTINATION, MUST BE TAPE:

DISK:, A:, B:, OR RUN: - REENTER

3.  NO FILE BY THAT NAME	INCORRECT NAME OR DRIVE - REENTER

ON DRIVE A

4.  CHECKSUM ERROR		CHECK SUM ON TAPE DOES NOT AGREE WITH

COMPUTED SUM. - VARIETY OF CAUSES

CHECK TERMINAL SPEED 4800 BAUD OR GREATER

REWIND TAPE TO BEGINNING OF FILE AND

TYPE CARRIAGE RETURN

5.  VERIFY ERROR		A BYTE IN THE TAPE FILE DOES NOT MATCH

THE DISK COPY - CHECK FOR INCORRECT TAPE

OR DISK

6.  ERROR - FILE TYPE NOT <COM>	ONLY COM TYPE FILES CAN BE EXECUTED - LOAD

- CANNOT BE RUN		FILE TO DISK AND RENAME

7.  DISK READ ERROR 		CP/M SYSTEM ERROR IN DISK READ

8.  DISK WRITE ERROR - DISK	DISK FILLED - CHANGE DISKS

OR DIRECTORY FULL

VII.  TAPE FORMAT

THE FORMAT FOR EACH RECORD ON THE TAPE IS AS FOLLOWS:

1.  START BYTES		03CH  (100)

2.  SYNC  BYTE		06EH

3.  DECODE BYTE		0FFH   (CONTROLS DISPLAY)

4.  TYPE  BYTE		0,1 OR 2

5.  LENGTH BYTE		NO OF 256 BYTE RECORDS

6.  NAME		CP/M FILE NAME  (11 BYTES)

7.  COMMENT		110 BYTES AVAILABLE BLANK IF NOT USED

8.  DATA		MULTIPLES OF 256 BYTE RECORDS

9.  CHECKSUM		ONE BYTE TARBELL CHECKSUM

IF A FILE IS UNBLOCKED IT WILL CONTAIN ONLY ONE CONTINUOUS

RECORD IN THE FORMAT GIVEN ABOVE. IF A FILE IS BLOCKED ONLY THE

FIRST RECORD OF THE FILE CONTAINS THE NAME AND COMMENT, THE REMAINING

FIELDS HOWEVER ARE THE SAME.

EACH FILE IS PRECEEDED BY A BLOCK OF 1500 START BYTES

FOLLOWED BY 100 HALT BYTES (76H). THE START BYTE BLOCK PROVIDES TIME

FOR DISK OPERATIONS TO COMPLETE AND THE HALT BYTES SERVE TO INDENTIFY

THE BEGINNING OF A FILE. THE NUMBER OF BYTES IN A TAPE RECORD IS

ALWAYS A MULTIPLE OF 256 NOT COUNTING THE START,SYNC,TYPE,LENGTH

AND DECODE BYTES. IF DATA DOES NOT EXACTLY FILL A RECORD, THE

REMAINDER OF THE RECORD IS FILLED WITH 1AH BYTES (THE CP/M END OF FILE

CODE).

THE BLOCKS OF A BLOCKED RECORD ARE SEPARATED BY APPROX 1 SECOND

BLOCKS OF START BYTES. TWO OF THE BLOCKS ARE APPROX 3 SECONDS LONG TO

ALLOW FOR THE EXTRA TIME REQUIRED IF THE FILE HAS MORE THAN ONE EXTENT.

VIII.  COMMENTS

SEVERAL POINTS SHOULD BE KEPT IN MIND WHEN USING TAPELIB

TAPE IS NOT NEARLY AS RELIABLE AS DISK FOR DATA STORAGE. ALWAYS

VERIFY TAPE COPIES OF DISK FILES BEFORE DESTROYING THE ORIGINALS.

USE REASONABLE QUALITY TAPE AND KEEP CASSETTE RECORDER HEADS CLEAN.

BECAUSE THERE IS NO DIRECTORY ON TAPES, BLOCK COPY OPERATIONS

FROM TAPE TO DISK DO NOT WORK AS RELIABLY AS DISK TO DISK COPIES.

THE PROGRAM HAS NO WAY OF TELLING WHEN ALL THE FILES ON A TAPE HAVE

BEEN FOUND. IT IS UP TO THE USER TO STOP THE TAPE AT THE PROPER

TIME WITHOUT PROMPTING FROM THE PROGRAM.

TAPE RECORDS ARE LABLED AND THEREFORE TAPE POSITIONING IS

NOT AT ALL CRITICAL. AS LONG AS THE TAPE STARTS BEFORE THE RECORD

REQUESTED THE PROGRAM WILL FIND THE FILE. THE PROGRAM MAY EVEN BE

STARTED WHILE READING LEADER.  HOWEVER IF TAPES ARE NOT ERASED

BEFORE USE, OLD FILES AT THE END OF THE TAPE MAY BE FOUND AND READ

DURING BLOCK COPY OPERATIONS. THE COPY-ALL OPERATION:

DISK:=*.*

IS THE WORST OFFENDER IN THIS RESPECT. IT IS RECOMMENDED THAT TAPES

BE COMPLETELY ERASED BEFORE BEING USED AS BACK UP FOR ALL THE

FILES ON A DISK.

CASSETTE RECORDERS ARE NOT CONTROLABLE DEVICES. IN GENERAL

THE PROGRAM HAS TO ACCEPT DATA AT THE FIXED SPEED COMING TO IT

FROM THE TAPE, 187 BYTES PER SECOND FOR TARBELL STANDARD. SUFFICIENT

SPACE MUST BE LEFT ON THE TAPE BETWEEN RECORDS TO ALLOW COPY TIME TO

DISK IF BLOCK COPIES ARE TO BE PERFORMED. ALTHOUGH LARGE GAPS ARE LEFT

ON THE TAPE BETWEEN FILES THE TIME MAY NOT BE SUFFICIENT IN ALL CASES.

IF THE TAPE DENSITY IS NOT TARBELL STANDARD OR DISK ERRORS OCCUR

REQUIRING MULTIPLE READS FOR ERROR CORRECTION ERRORS MAY OCCUR.

THE TYPE OF ERRORS WILL BE MISSED FILES DURING BLOCK COPIES OR MISSED

BLOCKS WHILE COPYING BLOCKED TAPES. SIMPLE INSPECTION WILL DETECT MISSING

FILES. SCAN THE TAPE AND COUNT THE RECORDS. MISSING BLOCKS ARE MORE

DIFFICULT BUT CAN USUALLY BE DETECTED BY REVERIFICATION OF THE TAPE.

IX.  TIMING DATA AND SPACE CONSIDERATIONS

DATA TRANSFER RATES (APPROX)

UNBLOCKED RECORDS		187 BYTES/SEC

BLOCKED RECORDS			140 BYTES/SEC

TIME BETWEEN FILES			8 SECONDS

TIME BETWEEN BLOCKS			0.85 SECONDS

LOAD TIME FROM TAPE FOR 10K BYTES

UNBLOCKED RECORD		72 SECONDS

BLOCKED RECORDS			83 SECONDS

ONE SIDE OF A C-60 TAPE WILL USUALLY STORE THE CONTENTS OF ONE

FLOPPY DISK.

############################