*************************************************************

*Readme type: Sample reference

*Readme file name: mandma.doc

*Library prefix: dma

*Library name: DMA

*Version: 1.00

*Creator: N.T.

*Created: 8/22/1994

*Comments: None

*************************************************************

1. Sample Description and Instructions

* smpdma0.c

[Title]

Low-level CPU DMA demo.

[Description]

Performs DMA transfers using CPU-based low-level functions. The

details of the transfer are shown below. DMA channels 0 and 1 are

used. Access the results of the transfer from address 0x6040000.

Source             Destination      Cnt

---------------    ---------------  ---

00000000 (BOOT) -> 06050000 (WORK)  200

00000200 (BOOT) -> 06050200 (WORK)  200

[Instructions]

(1) Initialize the boot ROM with the following commands

RS [Return]

GO [Return]

[CTRL-C]

GO 400 [Return]

[CTRL-C]

(2) Load smpper0.abs to address 6002000H and execute with GO

6002000 [Return].

[Functions Used]

DMA_CpuAllStop()

DMA_CpuSetComPrm()

DMA_CpuGetComStatus()

DMA_CpuGetStatus()

* smpdma1.c

[Title]

High-level CPU DMA demo.

[Description]

Performs DMA transfers using CPU-based high-level functions. The

details of the transfer are shown below. Access the results of the

transfer from address 0x6040000.

Source             Destination      Cnt

---------------    ---------------  ---

00000000 (BOOT) -> 06050000 (WORK)  100

[Instructions]

(1) Initialize the boot ROM with the following commands

RS [Return]

GO [Return]

[CTRL-C]

(2) Load smpper1.abs to address 6002000H and execute with GO

6002000 [Return].

[Functions Used]

DMA_CpuMemCopy1()

* smpdma2.c

[Title]

High-level CPU DMA demo

[Description]

Performs DMA transfers using CPU-based high-level functions. The

details of the transfer are shown below. Access the results of the

transfer from address 0x6040000.

WARNING:

Transfers with WORK RAM as the transfer source do not execute

correctly.

Byte Source             Destination      Cnt

---- ---------------    ---------------  -----

1    00000000 (BOOT) -> 06050000 (WORK)  100

2    00000000 (BOOT) -> 06050100 (WORK)  100

4    00000000 (BOOT) -> 06050300 (WORK)  100

16   00000000 (BOOT) -> 06050700 (WORK)  100

1    06050300 (WORK) -> 06060000 (WORK)  100

2    06050300 (WORK) -> 06060100 (WORK)  100

4    06050300 (WORK) -> 06060300 (WORK)  100

16   06050300 (WORK) -> 06060700 (WORK)  100

1    06050300 (WORK) -> 25c00000 (VDP1)  100

2    06050300 (WORK) -> 25c00100 (VDP1)  100

4    06050300 (WORK) -> 25c00300 (VDP1)  100

16   06050300 (WORK) -> 25c00700 (VDP1)  100

1    00000000 (BOOT) -> 25e00000 (VDP2)  80000

[Instructions]

(1) Initialize the boot ROM with the following commands

RS [Return]

GO [Return]

[CTRL-C]

(2) Load smpper2.abs to address 6002000H and execute with GO

6002000 [Return].

[Functions Used]

DMA_CpuMemCopy1()

DMA_CpuMemCopy2()

DMA_CpuMemCopy4()

DMA_CpuMemCopy16()

* smpdma10.c

[Title]

Low-level DMA demo.

[Description]

Performs DMA transfers using low-level DMA functions. The details

of the transfer are explained below. DMA channels 0, 1 and 2 are

used. Access the results of the transfer from address 0x6040000.

Source             Destination      Cnt

---------------    ---------------  ---

06050000 (WORK) -> 25c00000 (VDP1)  200

06050200 (WORK) -> 25c00200 (VDP1)  200

06050400 (WORK) -> 25c00400 (VDP1)  200

[Instructions]

(1) Initialize the boot ROM with the following commands

RS [Return]

GO [Return]

[CTRL-C]

(2) Load smpper10.abs to address 6002000H and execute with GO

6002000 [Return].

NOTE:

If the program size is small, code is executed only within the

cache. As a result, ICE commands become less effective. This is

normal and should not be considered as a bug.

[Functions Used]

DMA_ScuGetStatus()

DMA_ScuSetPrm()

DMA_ScuStart()

* smpdma12.c

[Title]

High-level DMA demo.

[Description]

Performs DMA transfers using high-level DMA functions. The details

of the transfer are shown below. Access the results of the

transfer from address 0x6040000.

Source             Destination      Cnt

---------------    ---------------  ---

06050000 (WORK) -> 25c00000 (VDP1)  500

[Instructions]

(1) Initialize the boot ROM with the following commands

RS [Return]

GO [Return]

[CTRL-C]

(2) Load smpper12.abs to address 6002000H and execute with GO

6002000 [Return].

NOTE:

If the program size is small, code is executed only within the

cache. As a result, ICE commands become less effective. This is

normal and should not be considered as a bug.

[Functions Used]

DMA_ScuMemCopy()

***************************end of file***************************