United States Air Force

Air Force Communications Command / Aerospace Rescue Service

OPLAN 9506 -- Search and Rescue

1/2

COMMUNICATIONS

During the coordination phase of requesting AFCC mobile

communication assistance, a joint AFCC/ARS determination

will be made regarding frequencies needed to support the

deployed unit. AFCC may elect to use MARS or other AFCC

available frequencies for point-to-point communications, or

may request MAC clearance of a different frequency.

Emergency Communications Frequencies

The unit in distress, or a station that has been assigned

controlling responsibility by the unit in distress, controls

distress traffic. However, for cases involving international

civil aviation, the station addressed by the distress

message controls distress traffic. Once communications are

established with a distressed unit, they should be

maintained on the same frequency. The following frequencies

have been assigned as distress or emergency frequencies:

500kHz  -- International CW/MCW distress & calling (Not

guarded by USN ships).   **Phasing out**

2182kHz -- International voice distress/safety/calling.

4125kHz -- International voice distress/safety/calling

backup (not guarded).

6215.5kHz- International voice distress/safety/calling

backup (not guarded).

8364kHz -- International CW/MCW lifeboat/survival craft &

SAR forces.

27.065mHz -- Citizens' Band Radio emergency freq. CB Ch# 9.

121.5mHz -- International VHF voice aeronautical emergency &

ELT/EPIRBs.

156.8mHz -- VHF-FM International voice distress/safety &

calling.  EPIRB alert tones. Marine channel 16.

156.75mHz-- Class C (marine) EPIRB 15 second homing signal.

243.0mHz -- Joint/Combined military UHF voice aeronautical

emergency, international survival craft & ELTs.

Search and Rescue Dedicated Frequencies

The following frequencies have been dedicated for SAR &

SAR coordination usage:

2670kHz -- USCG emergency coordination.

3023.5kHz -International voice SAR on-scene PRIMARY.

5680kHz -- International voice SAR on-scene.

8364kHz -- International survival craft & SAR on-scene.

Guarded by USN during aircraft Maritime Patrol

Operations in support of fleet units.

40.5mHz --(FM) United States DoD Joint Operations common.

121.6mHz --U.S./Canada voice SAR on-scene.

123.1mHz --International voice SAR on-scene PRIMARY.

138.45mHz -(FM) ARS on-scene.

138.78mHz -(FM) ARS on-scene (discrete).

156.3mHz -- Merchant ship/USCG on-scene. Marine radio ch# 6.

259.0mHz -- UHF/AM CONUS Air Rescue Operations.

282.8mHz -- Joint/Combined SAR DF & on-scene PRIMARY.

381.0mHz -- CONUS Air Rescue Operations.

381.8mHz -- USCG aircraft working frequency.

Search and Rescue COMMUNICATIONS

2/2

USCG Aircraft HF working freqs: (USB)

5692              8984

5696             11197

8980             11201

Civil Air Patrol (CAP)

4582kHz USB -- National Emergency & Calling frequency.

26.62mHz AM

143.90mHz AM/FM

148.15mHz AM/FM & FM repeaters.

SAR Mission Communications

The SAR Coordinator (SC) should establish the radio

frequencies available for assignment as control, on-scene,

monitor, homing, and press channels in the SAR Plan.  The

SAR Mission Coordinator (SMC) selects the SAR-dedicated

frequencies, informs the On-Scene Commander (OSC) and/or SAR

Units (SRUs) of the assigned frequencies, and establishes

communications between adjacent Rescue Coordination Centers

(RCCs) and with parent agencies of the units involved in the

SAR.  The OSC will maintain communications with all SRUs and

with the SMC. A primary & secondary frequency in the HF, VHF

and/or UHF band should be assigned as an "on-scene" channel.

The On-Scene Commander (OSC) controls the communications

on-scene and ensures that reliable communications are

maintained. The SRUs will report to the OSC on the assigned

"on-scene" frequency.

All units involved in the SAR operation should identify

themselves using vessel name, aircraft type or tail #, or

shore location or name.  Classified tactical callsigns

should not be used unless the SAR mission is classified or

behind enemy lines.  Armed Forces, CAP, or USCG aircraft

should use the word "rescue" in their callsign when priority

handling is critical.  SAR Units (SRUs) should initially

check in with the On-Scene Commander (OSC) using their full,

plain-language callsign. Thereafter, search area

assignments, such as "Alpha 6" or "Delta 2" should be used

as callsigns.

A Control Channel is to be used by the On-Scene

Commander & SAR Mission Coordinator only.

On Scene Channels are used between the OSC and SRUs.

Monitor Channels are guarded by the SRUs for possible

transmissions from the distressed craft/survivors.

En Route Channels are used by the SRUs and their parent

agencies until the SRU reaches the SAR scene, & switches

their operational control to the OSC.

A Homing Channel can be any On-Scene or Distress

frequency used for direction finding.

Press Channels are used by news media personnel for

filing stories. When possible, the Press Channels will be

regular HF/VHF/INMARSAT marine and/or aircraft frequencies

available to the public for phone patches, etc.

Primary CONUS RCC -- Scott AFB, IL.