| Data Code |  | Product Name | Oversea Switch |
| --- | --- | --- | --- |
| Obj.of use | Access engineer of oversea switch | Product Version | OVSV610R003B |
| Drawing Dept. | Oversea technical support Dept. of switch access network | Data version | Ver 1.0 |

Commissioning Guide for

Oversea Public Telephone

| Compiler： | Liu Feng | Date： | 2001/12/10 |
| --- | --- | --- | --- |
| Examiner & Checker： | Xia Wei | Date： | 2001/12/13 |
| Examiner & Checker： |  | Date： |  |
| Authrizer： |  | Date： |  |
| Record of Revision |  |  |  |
| Date | Revision Version | Author | Description |

Huawei Technologies Co., Ltd.

CONTENTS

| Commissioning Guide for Oversea Public Telephone | 3 |
| --- | --- |
| 1.1 model list of Public Telephone | 3 |
| 1.1.1 16KC Public Telephone | 3 |
| 1.1.2 12KC Public Telephone | 4 |
| 1.1.3 Public Telephone with reversed polarity | 4 |
| 1.1.4 Trunk Charging Pulse | 6 |
| 1.2 Data Configuration | 7 |
| 1.2.1 16/12KC Data Configure | 7 |
| 1.2.2 Data Configuration of Reversed Polarity | 7 |
| 1.2.3 Data Configuration of Trunk Pulse | 8 |
| 1.3 Hardware Requirement | 9 |

# Commissioning Guide for Oversea Public Telephone

As different country has different development level on telephone network, and most countries have the demand of coin phone. For China has rather advanced public telephone, so the engineer has few experience on commissioning of coin telephone, as well as its related hadware and software requirment, especially he is not familiar with trunk charging pulse. Now, we compiled this Commissioning Guide to guide oversea engineer to commission coin telephone, and to get some experience on trunk charging pulse.

1.1  Mode list of Public Telephone

So-called Public Telephone, usually denotes coin telephone,magnetic card telephone or IC telephone. The charging message transmits through subscriber line signaling, different public telephone uses different subscriber line signaling.

At present CC08 Exchange supports the following public telephones.

16KC

12KC

reversed polarity

pulse with reversed polarity

### 1.1.1  16KC Public Telephone

So-called 16KC Public Telephone, actually, according to configured charging matrix, MPU sends periodocally (usually 1 min) a series of pulses of 16KHZ frequency through subscriber board. Public telephone is charged according to the number of pulse series. Note that it’s the number of pulse series but not of pulses, pulse series are composed of a series of pulses.

.

16KC Public Telephone is divided into two kinds, one is KARLSON mode, another is HANDSON mode.

KARLSON mode, denotes that the pulse series are not sent periodocally, but sent as the call charge reaches a fixed value. For example,  in some place, the charging matrix of a pulse series is 5 cents, local charge is 5 cents per minute, trunk call 50 cents per minute. So when public telephone dials local call, firstly the exchange sends one pulse series, then sends one pulse series per a minute,  and when subscriber dials s trunk call, the switch sends a pulse series per 6 seconds.

ANDSON mode, denotes that pulse series are sent periodocally, as the above example, when subscriber dials a local call, firstly exchange  sends a pulse series which is like the KARLSON mode. When subscriber dials a trunk call,  then firstly exchange  sends 10 pulse series to public telephone, and sends 10 pulse series after 1minute.

The oversea basic version supports KARLSON mode in default, if office uses 16/12KC pulse telephone of KARLSON mode, please modify BIT0 of softeware parameter OVS_GLOBAL_PARA2 to “0”.

MOD SFP: ID=P118, VAL="fe";

## 1.1.2 12KC Public Telephone

12KC Public Telephone is the same as 16KC, except its pulse frequency is 12KHZ.

## 1.1.3   Public Telephone with Reversed Polarity

public telephone with reversed polarity，denotes that after judgement of the polarity reversion of subscriber’s line voltage, public telephone takes it as the base of charging.

Public telephone with reversed polarity has also two kinds, one is reversed polarity, another is pulse with reversed polarity.

Reversed polarity： telephone set with reversed polarity has own charding metrix, so only start and end time of call is necessary, exchange denotes the start and end time by polarity reversion on subscriber line.

Pulse with reversed polarity：this telephone set cann't charge itself. Telephone set will charge the user according to the polarity reversion pulses sent from switch. For example, when the called answers, its polarity reverses, as soon as call time is due, the polarity is restored after about 300MS. This reversion is called pulse with reversed polarity. If user continues to insert coin, the polarity reverses again to keep the call

Pulse with reversed polarity is divided into two kinds defined according to subscriber’s demand. One is pulse to be send when the cost reach the amount of coins. If user wants to keep talking, he should insert another coin. The other is pulse to be sent after subscriber’s answer or collecting the next coin, the call can be kept until switch sends next pulse with reversed polarity.

Three kinds of reversed polarities can be configured using command ADD MTRPLS, the subscriber set SEC PERIOD as 0 to denote reversed polarity billing, or else is pulse with reversed polarity billing. Otherwise is to set FIRST SEND POLARITY PULSE as NO to denote to send pulse with reversed polarity in the end of talk charge.

If reversed polarity is used,  it is not necessary to set second pulse as 0 using command ADD MTRPLS, so to the exchange only using reversed polarity, bit 8 of CCB parameter of oversea parameter of software parameter is applied to express reversed polarity, and its default value is reversed polarity, It is no use modifying MTRPLS now, reversed polarity is always used but not pulse with reversed polarity.

In addition, this command can be used whether switch sends alarm tone to public telephone subscriber before call charge ending.

The detailed description of polarity rexersion is as follows:

In each stage of talk, the conduct polarity of subscriber line (non-reversed polarity subscriber) must be:

“a” line refers to “-”

“b” line refers to “+”

To subscriber using pulse with reversed polarity，and sending pulse with reversed polarity to charge at the end of call, the conduct polarity of subscriber line must in compliance with Figure 1.

In the case of initial status, after on-hooking and dialing, listening call control signal and calling with urgent service station:

“a” line refers to “-”

“b” line refers to “+”                    （status1）

After the called answers and in talking（or paying from all kinds of coin telephone to paying service）;

“a” line refers to “+”

“b” line refers to “-”                                    （status2）

When paid talking time is due (e.g. 3 minutes), after short time polarity restore with time length 300+/-50ms（status1），then it turns to status2 so as to append paying and continue talk (denotes local communication coin telephone able to perform talk delay and call time limiting ).

When a subscriber on-hooks－－turn to status1（polarity restore）

Figure 1.

| Initial status Subscriber on-hook | Subscriber B answer Polarity reversion |  | 20ms prompt signal | Short-time restore of polarity |
| --- | --- | --- | --- | --- |
| answer |  |  |  | 300 ms |
| dialing |  |  |  |  |
| Call sending control |  |  |  |  |
| －"a" | ＋ |  |  | － |
| ＋ “b” | － |  |  | ＋ |
|  | collect the 1st coin | 3 min |  | Collect the 2nd coin |

|  | 20s prompt signal | Short-time restore of polarity | Polarity reversion Hear in coin phone | A subscriber on-hook Restore to initial polarity |
| --- | --- | --- | --- | --- |
| ＋ | Pay one coin talk charge | 300 ms － | ＋ | － |
| － |  | ＋ | － | ＋ |
|  | 3 min | No coin talk |  |  |

For subscriber in use of pulse with reversed polarity，who sends pulse with reversed polarity at the end of call charge，its conduct polarity of subscriber wire must in compliance with figure 2.

Figure2

| Initial status Subscriber on-hook dialing | Subscriber answer 1st charging pulse short-time reversion of polarity 300 ms | Push “answer” key | From the 1st charging pulse on pay one coin talk |
| --- | --- | --- | --- |
| －"a" | + | － |  |
| ＋"b" | － | ＋ |  |
|  | Coin phone into work status | collect first coin | Per billing range |
| The 2nd charging pulse： short-time reversion of polarity 300 ms | Pay the 2nd coin talk | Talk without coin 3-6s | Auto on-hook at coin phone ending |
| + | － | － |  |
| － | ＋ | ＋ |  |
| Collect 2nd coin | per billing range |  |  |

### 1.1.4  Trunk Charging Pulse

Trunk charging pulse, denotes part of end office not enable to trunk billing to which upper-level exchange sends charging pulse to public telephone of end office through  trunk. Setting of “use oppsite pulse” has different meaning for outgoing and incoming trunk. For outgoing trunk, it denotes to receive charging pulse from opposite exchange ,and the opposite exchange must send charging pulse in match; For incoming trunk, it denotes to receive charging pulse from opposite exchange in incoming call of  own truk group, meanwhile, opposite exchange will receive charging pulse in match

“use oppsite pulse” will be configured in trunk group attribute But the opposite exchange may determine sending charging pulse or not according to the calling type of our office. For ISUP message, MPM or CRG message can be observed by signaling trace, which denotes the opposite exchange has sent charging pulse. If there is no MPM or CRG message, then the opposite exchange doesn’t sent charging pulse. For CAS signaling,  the opposite exchange send actual pulse, which can not be observed from NMS, so the only method is to test by terminal phone set. In addition, there are more meaning about CRG message, for understanding those message, pls refers to documents CRG Message Billing Description”

## Data Configuration

### 1.2.1 16/12KC Data Configuration

The Charging Data Configuration is the same as ordinary subscriber

Set the subscriber as immediate subscriber table

MOD ST: D=K'8888888, CHT=IMU, PLST=KHZ12;

Modify the rate of KC16/12 pulse as actual rate again,  here supposed as 10 .

MOD MTRR: MID=KC12, RAT=10;

Switch can calculate the pulse number needed to be send to public telephone set automatically according to rate of KC16/12 in Data Configuration.

The value of pulse charging tariff should not be '0'

Note should be taken that not to set dialed calee as free call, otherwise the pulse can not besent.

### 1.2.2   Data Configuration with Reversed Polarity

User data configuration with reversed polarity is more simple.

If the exchange is pulse with reversed polarity, then use the command MOD SFP to modify BIT 8 of oversea CCB parameter 1 to 0. The parameter means:

Default value is 1: which means the subscriber with pure reversed polarity, in this time only subscriber charging pulse is modified as reversed polarit, thus the subscriber is the subscriber with pure reversed polarity.

Is 0: which means the subscriber with reversed polarity, in this time subscriber charging pulse is modified as reversed polarit, charging mode as immediate subscriber list, add charging pulse data, then the subscriber can realize pulse with reversed polarity according to charging pulse data as firstly sent pulse or pulse with reversed polarity.

Charging data is set according to actual situation:

Modify subscriber as subscriber with reversed polarity.

MOD ST: CHT=IMU, PLST=RVS;

Then configuring data according to customer requirment.

ADD MTRPLS: CHSC=0, RCHS=0, FPRD=180, SPRD=180;

The above command means that CHARGING SOURCE is 0, CHARGING SELECTION is 0. The first and successive coins can call 180S, when near the end of call charge send pulse with reversed polarity (default configuration).

If SPRD is set as 0, then it is subscriber with reversed polarity (pulse is not sent).

Note that don’t to set called number as free call, otherwise the pulse can not be sent.

### 1.2.3  Data Configuration of trunk charing pulse

Firstly, trunk pulse is not limited to public telephone billing, it can also be applied to ordinary subscriber’s billing by receiving trunk charging pulse, or for subscriber billing of opposite terminal exchange through sending trunk charging pulse.

Data Configuration method of oversea basic version before 2K (except 2K) version.

Configure billing data according to actual situation:

Modify trunk group table to make it possible to receive trunk pulse :

MOD N1TG: TG=0, UPF=YES;

Configure the rate of trunk charging pulse

MOD MTRR: MID=TKCHGPLS, RAT=10;

Note:

Meaning of trunk charging pulse is quite differrent for incoming and outgoing trunk. In the case of outgoing trunk, it means that the trunk group receive charging pulse from opposite exchange during outgoing call, naturally the opposite exchange should send charging pulse in match; while in the case of incoming trunk, it means that the trunk group send charging pulse to opposite exchange during incoming call, and it means only in the case that in this time the opposite exchange receive charging pulse in match

The rate of trunk charging pulse mustn’t be '0'.

NO7 trunk has no trunk charging pulse, in replacement, it has CRG or MPM message which are used for charging information. Because of the complexity of CRG message (different for each country) . the message is not fully supported. If there is CRG message demand during the software commissioning,  please consult to company head quarter for adaptive data.

Note: donot set called number as free, otherwise the trunk charging pulse can not be sent.

Data Configuration Method of Oversea Basic Version after 2K (including 2K) version.

As Oversea Basic Version before 2K can’t process CRG message flexibly, version 2K modified billing data to make trunk charging pulse be processed more flexible. For detail description of CRG , please refer to CRG Message Description”. Here we will not explain more. In the following we explain Data Configuration Method only.

Charging Analysis Table: field CHARGING MANNER and field CHARGING BAND NUMBER are newly added in the table. Field CHARGING MANNER is CRG billing form, including non-CRG form, Intra-office CRG form, field no receiving  CRG billing, field no sending  CRG billing etc. the default value of CHARGING MANNER is NON CHARGING MODE denotes charged by Switch itself, if only CRG message applied to transmit trunk charging pulse, set field CHARGING MANNER as [RECEIVE CHARGING PULSE MPM]. if only sending trunk charging pulse, then set it as [SEND CHARGING PULSE MPM],  other fields are not explained temporarily.

Destination Code charging Table: Key field BILL TYPE is added in the table, which may take values REGULAR（periodically），METER（immediate meter），PRINT（immediate print），FREE，ALL BILL TYPE.Addition of this field mainly is based on the following cause, that for the same charging selection code and charging source cade,  case may exists that ordinary subscriber needs not to receive or send pulse, only public telephone receives or send charging pulse, then it can be differentiated by Destination Code Charging Table. The default value of this field is ALL BILL TYPE.

For CCS7 signaling, parameter [USE OPPOSITE PULSE] of NO7 trunk group will not function; and for CAS signaling,【USE OPPOSITE PULSE 】data will still function with a different meaning as before.

Before,  【USE OPPOSITE PULSE】is TRUE， for incoming trunk it means that switch surely will send charging pulse, and for outgoing trunk it means that switch surely will receive charging pulse;

Now it only means MAY receive or send metering pulse; the cause to add these two parameter is that, board is sent a command to start receiving pulse from CAS outgoing trunk; and for incoming trunk, answer signal is replaced by MPM message. When tha call ended, these two parameter is used to send stop command. In addition, Some country consider answer signal as the first charging pulse, that may realized by modifying bit 7 of OVS_CAS_PARA1 to 0.

## Hardware Requirement

All ports of CB36ASL, CB13ASL support all above types of Public Telephone,  at present mainly CB36ASL is applied.

CC08ASL supports reversed polarity subscriber only in 8th and 9th ports (counting from 1) on board subscriber, but not KC16/12 and subscriber of pulse with reversed polarity.